EXPERIMENT 3 – ADC

MATLAB-Based Modelling of VSB Modulator and Demodulator for Analog Television Communication Applications

clc;

clear;

close all;

%% VSB-SC MODULATOR AND DEMODULATOR

%% 1. Parameters

Fs = 500000;

T = 0.02;

t = 0:1/Fs:T-1/Fs;

N = length(t);

fc = 100000;       % Carrier frequency

Ac = 1;            % Carrier amplitude

fm1 = 2000;

fm2 = 5000;

fm3 = 8000;

%% 2. Message / TV Video Signal

m = 0.5*cos(2*pi*fm1*t) + ...

0.3*cos(2*pi*fm2*t) + ...

0.2*cos(2*pi*fm3*t);

m = m / max(abs(m));

%% 3. Carrier

carrier = Ac*cos(2*pi*fc*t);

%% 4. DSB-SC Modulation

dsb_sc = m .* carrier;

%% 5. Frequency Axis

f = (-N/2:N/2-1)*(Fs/N);

%% 6. DSB-SC Spectrum

DSB = fftshift(fft(dsb_sc));

%% 7. VSB Filter

Bm = fm3;

Bv = 2000;

H = zeros(size(f));

% Positive frequencies

H(f >= fc & f <= fc + Bm) = 1;

H(f >= fc-Bv & f < fc) = 0.5;

% Negative frequencies

H(f <= -fc & f >= -fc-Bm) = 1;

H(f < -fc & f >= -fc+Bv) = 0.5;

%% 8. VSB Signal

VSB_spectrum = DSB .* H;

vsb = real(ifft(ifftshift(VSB_spectrum)));

%% 9. Add Noise

SNR_dB = 30;

signal_power = mean(vsb.^2);

noise_power = signal_power / (10^(SNR_dB/10));

noise = sqrt(noise_power) * randn(size(vsb));

received_vsb = vsb + noise;

%% 10. Coherent Detection

local_carrier = 2*cos(2*pi*fc*t);

detector_output = received_vsb .* local_carrier;

%% 11. Low Pass Filter

cutoff = Bm;

[b,a] = butter(6, cutoff/(Fs/2));

recovered_signal = filtfilt(b,a,detector_output);

%% 12. Normalize

recovered_signal = recovered_signal / max(abs(recovered_signal));

%% 13. Performance Analysis

MSE = mean((m - recovered_signal).^2);

R = corrcoef(m,recovered_signal);

Correlation = R(1,2);

signal_power = mean(m.^2);

error_power = mean((m-recovered_signal).^2);

SNR = 10*log10(signal_power/error_power);

%% 14. Display Results

fprintf('\n====================================\n');

fprintf('       VSB SYSTEM RESULTS\n');

fprintf('====================================\n');

fprintf('Carrier Frequency       = %.2f kHz\n',fc/1000);

fprintf('Message Bandwidth       = %.2f kHz\n',Bm/1000);

fprintf('Vestigial Bandwidth     = %.2f kHz\n',Bv/1000);

fprintf('VSB Bandwidth           = %.2f kHz\n',(Bm+Bv)/1000);

fprintf('\nPerformance:\n');

fprintf('MSE                     = %.6f\n',MSE);

fprintf('Correlation Coefficient = %.4f\n',Correlation);

fprintf('SNR                     = %.2f dB\n',SNR);

fprintf('====================================\n');

%% 15. Plot Original Signal

figure;

plot(t*1000,m,'LineWidth',1.2);

xlabel('Time (ms)');

ylabel('Amplitude');

title('Original TV Video Signal');

grid on;

xlim([0 2]);

%% 16. Plot DSB-SC Signal

figure;

plot(t*1000,dsb_sc);

xlabel('Time (ms)');

ylabel('Amplitude');

title('DSB-SC Signal');

grid on;

xlim([0 0.5]);

%% 17. DSB-SC Spectrum

figure;

plot(f/1000,abs(DSB)/N);

xlabel('Frequency (kHz)');

ylabel('Magnitude');

title('DSB-SC Spectrum');

grid on;

xlim([-120 120]);

%% 18. VSB Filter Response

figure;

plot(f/1000,H,'LineWidth',1.5);

xlabel('Frequency (kHz)');

ylabel('Gain');

title('VSB Filter Response');

grid on;

xlim([-120 120]);

%% 19. VSB Spectrum

figure;

plot(f/1000,abs(VSB_spectrum)/N);

xlabel('Frequency (kHz)');

ylabel('Magnitude');

title('VSB-SC Spectrum');

grid on;

xlim([-120 120]);

%% 20. Received VSB Signal

figure;

plot(t*1000,received_vsb);

xlabel('Time (ms)');

ylabel('Amplitude');

title('Received VSB Signal');

grid on;

xlim([0 0.5]);

%% 21. Product Detector Output

figure;

plot(t*1000,detector_output);

xlabel('Time (ms)');

ylabel('Amplitude');

title('Product Detector Output');

grid on;

xlim([0 2]);

%% 22. Recovered Signal

figure;

plot(t*1000,recovered_signal,'LineWidth',1.2);

xlabel('Time (ms)');

ylabel('Amplitude');

title('Recovered TV Signal');

grid on;

xlim([0 2]);

%% 23. Original vs Recovered

figure;

plot(t*1000,m,'LineWidth',1.2);

hold on;

plot(t*1000,recovered_signal,'--','LineWidth',1.2);

xlabel('Time (ms)');

ylabel('Amplitude');

title('Original vs Recovered TV Signal');

legend('Original Signal','Recovered Signal');

grid on;

xlim([0 2]);